# Van’s Aircraft RV-6A

# Pilot’s Operating Handbook

Make: Allen R. Checca

Model:  RV-6A

Serial: 60573

Registration: N468AC

PERFORMANCE – SPECIFICATIONS

SPAN: ………………………………………………………………..	23’ 0”

LENGTH: …………………………………………………………….	19’ 9”

HEIGHT:  …………………………………………………………….	6’ 8”

SPEED:

Maximum at Sea Level ……………………………………..	182 Knots

Cruise, 75% Power at 8,000 Ft ……………………………..	167 Knots

RANGE (includes 3 gal. for taxi, takeoff & climb):

75% @ 8000’, no reserve.………………………………….	695 nm

55% @ 8000’, no reserve ….……………………………….	840 nm

75% @ 8000’, one hour (8 gal) reserve ……………………..	540 nm

55% @ 8000’, one hour (8 gal) reserve ……………………..	650 nm

RATE OF CLIMB AT SEA LEVEL (At 1650 Lbs / Standard day).	2,050 FPM

SERVICE CEILING …………………………………………..…….	19,500 FT

TAKEOFF PERFORMANCE: (At 1650 Lbs / Standard day).………	535 Ft

LANDING PERFORMANCE: (At 1650 Lbs / Standard day).………	500 Ft

STALL SPEED (CAS):

Flaps Up, Power Off ………………………………………	60 Knots

Flaps Down, Power Off ……………………………………	56 Knots

MAXIMUM WEIGHT (Normal Category): ………………………….	1900 Lbs

MAXIMUM LANDING WEIGHT (Normal Category):……………..	1800 Lbs

MAXIMUM LANDING WEIGHT (Soft Field):……………………..	1650 Lbs

EMPTY WEIGHT ………………………………………….		1152 Lbs

MAXIMUM USEFUL LOAD: ……………………………………..	748 Lbs

BAGGAGE ALLOWANCE …………………………………………	100 Lbs

WING LOADING (Pounds/ Sq. Ft) …………………………………	17.27

POWER LOADING (Pounds/ HP) ………………………………….	10.55

FUEL:

Capacity ……………………………………………………	38 Gal Total

Type ……………………………………………………….	100 LL

OIL CAPACITY …………………………………………………….	8 Qts

ENGINE: Lycoming ………………………………………………..	O-360-A1A

PROPELLER: Senisentch …………………………………….   	72FM859-1-85

AIRSPEED LIMITATIONS

|  | SPEED | KIAS | REMARKS |
| --- | --- | --- | --- |
| VNE | Never Exceed Speed | 182 Kts | Do not exceed this speed in any operations. |
| #### VNO | Maximum Structural Cruising Speed | 156 Kts 135 Kts | 1650 Lbs 1900 Lbs Exceed this speed only in smooth air. |
| VA | Maneuvering Speed | 115 Kts 100 Kts | At 1650 Lbs At 1900 Lbs Full control deflection generates 6 g’s |
| VFE | Maximum Flap Extended Speed | 96 –20 deg 87 - Full | Do not exceed this speed with flaps down |
| Vy | Best Rate of Climb | #### 103 Kts | 120 Kts best climb/cooling speed |
| Vx | Best Angle of Climb | 65 Kts |  |
| Vs | Stall Speed Clean | 60 Kts |  |
| Vso | Stall Speed Landing Configuration | 56 Kts |  |

AIRSPEED INDICATOR MARKINGS

| MARKING | KIAS VALUE OR RANGE | SIGNIFICANCE |
| --- | --- | --- |
| White Arc | 43 – 87 Kts | Full Flap Operating Range. Lower limit is Vso. Upper limit is maximum speed with flaps extended |
| Green Arc | 47 – 156 Kts | Normal Operating Range. Lower limit is Vs. Upper limit is maximum structural cruising speed |
| Yellow Arc | 156 – 182 Kts | Operations must be conducted with caution and only in smooth air. |
| Red Line | 182 Kts | Maximum speed for all operations |

## Views

##### AEROBATIC INFORMATION

Weight Limitation – 1375 Pounds (aerobatic) +6, -6 g

| Pilot, No Pax, No Baggage |  | ******** | Aerobatic |
| --- | --- | --- | --- |
| Item | Weight | Arm | Moment |
| Aircraft | 1152 |  | 80518.14 |
| Fuel (6 lbs/gal) | 60 (10 Gallons) | 70.0 | 4200 |
| Pilot | 165 | 87.4 | 14421 |
| Passenger | 0 | 87.4 | 0 |
| Baggage | 0 | 117 | 0 |
| Total | 1375 |  | 99139.14 |
| CG: | 72.00 |  |  |
| Zero Fuel CG | 72.09 |  |  |

| Aerobatic CG Range: | 68.70 | to | 75.30 |
| --- | --- | --- | --- |

Recommended Entry Speeds:

Loops, Horizontal Eights 	120 – 165 Kts

Immelman Turns 		130 – 165 Kts

Aileron Rolls, Barrel Rolls 	105 – 165 Kts

Snap Rolls			70  –   95 Kts

Vertical Rolls 			155 –165 Kts

Split-S 			85  -  95 Kts

THIS AIRCRAFT IS APPROVED FOR POSITIVE g AEROBATIC OPERATIONS.  INTENTIONAL SPINS ARE NOT RECOMMENDED, AS AIRSPEED TENDS TO BUILD RAPIDLY IN THIS AIRCRAFT.

##### PREFLIGHT INSPECTION

1.	CABIN

- Documentation -- Available In Airplane

- Aeronautical Charts – CURRENT AND APPROPRIATE TO FLIGHT

- Seat Belt Securing Control Stick --  RELEASE

- Control locks -- REMOVED

- Ignition Switch  -- OFF

- Avionics -- OFF

- Master Switch -- ON

- Flaps – DOWN

- Master Switch -- OFF

- Carb Heat – COLD

2. EMPENNAGE

- Tail Tie-Down -- DISCONNECT

- Control Surfaces -- CHECK freedom of movement and security

- Elevator trim – CHECK take off position

- Static Sources (both sides of fuselage) –CHECK for blockage

3. RIGHT WING

- Aileron -- CHECK freedom of movement and security

- Flap -- CHECK security

- Landing light – CHECK condition

- Wing Tie-Down -- DISCONNECT

- Main Wheel Tire -- CHECK for 45#

- Chock -- REMOVE

- Right Wing Tank – SUMP

- Fuel Quantity -- CHECK VISUALLY

- Fuel Filler Cap – SECURE

5. NOSE

- Engine Oil Level -- CHECK, do not operate with less than 4 quarts

- Propeller and Spinner -- CHECK for nicks and security

- Cowl Hinge Pins – CHECK for security

- Air Inlet -- CHECK for restrictions

- Nose Wheel Tire -- CHECK for 40#

- Chock -- REMOVE

- Fuel Tank Vents -- CHECK for blockage

6. LEFT WING

- Wing Tie-Down -- DISCONNECT

- Main Wheel Tire -- CHECK for 45#

- Chock – REMOVE

- Left Wing Tank -- SUMP

- Fuel Quantity -- CHECK VISUALLY

- Fuel Filler Cap -- SECURE

- Pitot Tube Cover -- REMOVE and check for blockage

- Landing Light -- CHECK condition

- Aileron -- CHECK freedom of movement and security

- Flap -- CHECK security

#### BEFORE STARTING ENGINE

- Preflight Inspection -- COMPLETE

- Seat Belts and Shoulder Harnesses -- ADJUST and LOCK

- Fuel Selector Valve – MOST FULL TANK

- Avionics and Electrical -- OFF

- Brakes -- SET

- Circuit Breakers -- CHECK IN

- Canopy -- ADJUST

#### STARTING ENGINE (cold)

- Mixture -- RICH

- Throttle to 1/8

- Master Switch – ON

- Flaps -- UP

- Fuel Boost Pump – ON

- Prime – Mix to rich (4-5 seconds when cold)

- Propeller Area -- CLEAR

- Ignition Switch -- START

- Avionics & Instruments – ON

- Oil Pressure -- CHECK   25  psi at idle

- Position lights – ON

### STARTING ENGINE (Warm)

- Mixture -- RICH

- Throttle to 1/8

- Master Switch – ON

- Flaps -- UP

- Fuel Boost Pump – ON (5-10 seconds to purge)

- Fuel Boost Pump -- OFF

- Propeller Area -- CLEAR

- Ignition Switch -- START

- Avionics & Instruments -- ON

- Oil Pressure -- CHECK   25  psi at idle

- Position lights – ON

BEFORE TAKEOFF

- Brakes -- SET

- Flight Controls -- FREE and CORRECT

- Flight Instruments – SET
	Altimeter – CORRECT PRESSURE
	D/G – CORRECT HEADING
	GPS ON

Flight timer – RESET

Hobbs time -- NOTE

- Fuel Selector Valve – MOST FULL TANK

- Circuit breakers – RESET

- Auto pilot – ON - DISABLED

- Mixture -- RICH (below 3000’)

- Elevator Trim – NEUTRAL

- Flaps -- UP

- Throttle -- 1700 RPM

- Magnetos --  CHECK (125 max drop, 50 differential)

- Magnetos --  EGT Raise on all Cyl

- Carb heat -- CHECK

- Engine Instruments – NO ALARMS

- Throttle -- IDLE

- Radios -- SET

- Fuel Boost Pump -- ON

- Transponder – ALTITUDE

- Passenger – READY and willing

- Strobe lights – ON

- Sunshade – RETRACT

- Canopy  -------- Main Latch – SECURE

##### TAKEOFF

#### NORMAL TAKEOFF

- Wing Flaps – UP

- Throttle --  FULL OPEN

- Elevator Control – LIFT NOSE WHEEL (at 55KIAS)

- Climb Speed --  120 KIAS

#### SHORT FIELD TAKEOFF

- Wing Flaps – 5 Deg

- Brakes – APPLY

- Throttle – FULL OPEN

- Mixture – RICH (above 3000’ lean to obtain max RPM)

- Brakes – RELEASE

- Climb Speed – 80 KIAS (Vy)

##### ENROUTE CLIMB

- Airspeed – 120 - 140 KIAS

- Throttle – 25 in Hg or full throttle or 2400 RPM or 75% power

- Boost Pump – ON

- Fuel Pressure – CHECK

- Mixture – LEAN above 5000’

##### CRUISE

- Throttle – 23 in Hg or 2300 RPM or less than 75% power

- Trim – ADJUST

- Mixture – LEAN to 50 deg C rich of peak

##### DESCENT

a)   Start descent 6 miles out for every 1000 feet

b)   Trim – ADJUST to 500 feet per minute

c)   Mixture – FULL RICH

d)   Throttle – REDUCE manifold pressure 1” every 2 minutes

##### LANDING

- Fuel – SELECT most full tank

- Sunshade -- RETRACT

- Mixture – FULL RICH

- Fuel boost pump – ON

- All lights – ON

- Carb heat – ON

- Speed – SLOW to 86 kts

- Flaps -- Full

- Elevator trim – 86 kts

##### POSTFLIGHT

##### AFTER LANDING

- Wing Flaps – UP 1”

- Boost Pump – OFF

- Transponder – STANDBY

ENGINE SHUTDOWN

- Flaps – DOWN until 1” travel remains

- Throttle – IDLE

- CHT decidedly dropped

- All electrical switches  – OFF

- Avionics and Instruments.-- OFF

- Mixture – IDLE CUT-OFF

- Wait for shut down

- Ignition -- OFF

- Master – OFF

#### SECURING AIRCRAFT

- Wheel Chocks

- Wing & Tail Tie-Down

- Pitot Tube Cover

- Cockpit control lock -- SECURED

- Ignition Key – REMOVED

- Master and Electrical Switches – OFF

##### Performance

Cruise Performance at 8,000’:

| KTAS | RPM | MAP | Fuel Flow | % Power |
| --- | --- | --- | --- | --- |
| 167 | 2400 | 23” | 8.4 GPH | 75% |
| 157 | 2300 | 22” | 7.4 GPH | 65% |
| 151 | 2200 | 21” | 6.3 GPH | 55% |

No Wind Range at 8,000’:

- All range calculations include 3 gal. For engine start, taxi, takeoff and climb.

- Engine is leaned for best economy.

One hour (8 gal.) reserve:

75% Power	540 nm

65% Power	575 nm

55% Power	650 nm

No Reserve:

75% Power	695 nm

65% Power	745 nm

55% Power	840 nm

KINDS OF OPERATING LIMITS

The RV6A – N468AC airplane is approved for the following operations when equipped in accordance with FAR 91.205

(a) Day V.F.R.

(b) Night V.F.R.

(c) Day I.F.R.

(d) Night I.F.R.

(e) Non Icing

(f) Acceleration limits

at 1900 Lbs +2, -1.5 g 
   	at 1650 Lbs +6, -4 g 
   	at 1375 Lbs (aerobatic) +6, -6 g

FLAP LIMITATIONS

Approved Takeoff Range: UP to 5°

Approved Landing Range: UP to FULL

FUEL LIMITATIONS

(a) Total Capacity 38 U.S. GAL

(b) Unusable Fuel 0 U.S. GAL

The unusable fuel for this airplane has been determined as 0.0 gallon in each wing in critical flight attitudes.

(c)Usable Fuel 38 U.S. GAL

The usable fuel in this airplane has been determined as 19.0 gallons in each wing.

##### WEIGHT AND BALANCE DATA

Make: Allen Checca			Model:  RV-6A

Serial: 60573				Registration: N468AC

Maximum Weights:

Aerobatic Category …………………………	1375 Lbs

Utility Category …………………………….	1650 Lbs

Normal Category ……………………………	1900 Lbs

Datum= 60 inches forward of wing leading edge (L.E.)

Design C.G. Range = 	15% to 29% of wing chord

8.7” to 16.8” from L.E.

68.7” to 77.1” aft of Datum

Wing L.E. = 60 inches aft of datum

Main wheel right = 84.25” aft of datum

Main wheel left =   84.25” aft of datum

Nose wheel =          28.50” aft of datum

Fuel ……………………	70” aft of datum

Pilot and Passenger ……	87.4” aft of datum

Baggage ……………….	117” aft of datum

Aircraft weighed empty in level flight attitude.

(Includes 8 qts. of oil, no useful fuel)

Sample:

Weight		Arm		Moment

Aircraft	1152				80518.14

Fuel (6lbs/gal)			70

Pilot				87.4

Passenger			87.4

Baggage			117

Total

CG = Total Moment / Total Weight

CG = 		 in aft of datum

CG Range = 68.7 to 77.1 in aft of datum

Empty CG = 69.89

##### WEIGHT AND BALANCE DATA

*All CG ranges for possible flight conditions are within design limits

|  | Left Main | Rt. Main | Nose |  | Total |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Weight | 427 | 429 | 296 |  | 1152 |  |  |  |  |  |  |  |  |
| Component | Weight | * | Arm | = | Moment |  |  |  |  |  |  |  |  |
| Lmain | 427 |  | 84.13 |  | 35974.75 |  |  |  |  |  |  |  |  |
| Rmain | 429 |  | 84.25 |  | 36089.63 |  |  |  |  |  |  |  |  |
| Nose | 296 |  | 28.56 |  | 8453.76 |  |  |  |  |  |  |  |  |
| sub total |  |  |  |  |  |  | 80518.14 |  |  |  |  |  |  |
| CG Range | 68.70 to 76.80 in aft of datum |  |  |  |  |  |  |  |  |  |  |  |  |
| Empty C.G | 69.89 |  |  |  |  |  |  |  |  |  |  |  |  |
| Max. Gross Weight |  |  |  |  | Full Baggage, No Pax |  |  |  |  |  |  |  |  |
| Item | Weight | Arm | Moment |  |  | Item |  | Weight |  | Arm |  | Moment |  |
| Aircraft | 1152 |  | 80518.14 |  |  | Aircraft |  | 1152 |  |  |  | 80518.14 |  |
| Fuel (6 lbs/gal) | 228 | 70 | 15,960 |  |  | Fuel (6 lbs/gal) |  | 228 |  | 70 |  | 15,960 |  |
| Pilot | 200 | 87.4 | 17480 |  |  | Pilot |  | 250 |  | 87.4 |  | 21850 |  |
| Passenger | 200 | 87.4 | 17480 |  |  | Passenger |  | 0 |  | 87.4 |  | 0 |  |
| Baggage | 100 | 117 | 11700 |  |  | Baggage |  | 100 |  | 117 |  | 11700 |  |
| Total | 1900 |  | 143138.14 |  |  | Total |  | 1730 |  |  |  | 130028.14 |  |
| CG: | 76.14 |  |  |  |  | CG: |  | 75.16 |  |  |  |  |  |
| Zero Fuel CG | 76.98 |  |  |  |  | Zero Fuel CG |  | 75.94 |  |  |  |  |  |
| Pilot, Pax, No Baggage |  |  |  |  | Pilot, No Pax, No Baggage |  |  |  | ******** |  | Aerobatic |  |  |
| Item | Weight | Arm | Moment |  |  | Item |  | Weight |  | Arm |  | Moment |  |
| Aircraft | 1152 |  | 80518.14 |  |  | Aircraft |  | 1152 |  |  |  | 80518.14 |  |
| Fuel (6 lbs/gal) | 228 | 70 | 15,960 |  |  | Fuel (6 lbs/gal) |  | 60 |  | 70.0 |  | 4200 |  |
| Pilot | 250 | 87.4 | 21850 |  |  | Pilot |  | 165 |  | 87.4 |  | 14421 |  |
| Passenger | 250 | 87.4 | 21850 |  |  | Passenger |  | 0 |  | 87.4 |  | 0 |  |
| Baggage | 0 | 117 | 0 |  |  | Baggage |  | 0 |  | 117 |  | 0 |  |
| Total | 1880 |  | 140178.14 |  |  | Total |  | 1375 |  |  |  | 99139.14 |  |
| CG: | 74.56 |  |  |  |  | CG: |  | 72.00 |  |  |  |  |  |
| Zero Fuel CG | 75.19 |  |  |  |  | Zero Fuel CG |  | 72.09 |  |  |  |  |  |

MAXIMUM TAKE OFF WEIGHT:			1900 Lbs

MAXIMUM LANDING WEIGHT:			1800 Lbs

MAXIMUM LANDING WEIGHT (Soft Field):	1650 Lbs

MAXIMUM SEAT WEIGHT:			250 Lbs

MAXIMUM FUEL WEIGHT:			228 Lbs

##### Engine Information

Model: ……..	Lycoming O-360 A1A

HP: …………	180

Fuel: ………..	91/96 or 100/130 octane minimum

………..	100LL

Oil Filter: …..	Champion CH48110

OIL:  Avg	MIL-L-6082	Ashless Dispersant

Ambiant Air	   Grades		     Grades

Above 80F		SAE 60		      SAE 60,20w50

Above 60F		SAE 60		      SAE 60

30 – 90F		SAE 40		      SAE 40,50

0-70F			SAE 30		      SAE 30,40 or SAE 40

###### Below 10F			SAE 20		      SAE 30 or 20w30

###### Oil Sump Capacity 8 U.S. Quarts

###### Minimum Safe Quantity 2 U.S. Quarts

###### Operating Conditions:

###### Oil Inlet Temp: 185 deg F desired, 245 deg F Maximum

###### Oil Pressure:		95 psi max; 	 55 psi min; 	 25 psi idle

###### Fuel Pressure:	8 psi max;	 1 psi min;	 4 psi desired

###### Cyl. Head Temp 150 deg F – 400 deg F desired range,  500 deg F max

###### Max oil consumption .45qts/hr. at cruise

###### Conversion table:

| Celc. | Fahr. | Celc. | Fahr. | Celc. | Fahr. | Celc. | Fahr. | Celc. | Fahr. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 50 | 60 | 140 | 110 | 230 | 160 | 320 | 210 | 410 |
| 16 | 60 | 66 | 150 | 116 | 240 | 166 | 330 | 216 | 420 |
| 21 | 70 | 71 | 160 | 121 | 250 | 171 | 340 | 221 | 430 |
| 27 | 80 | 77 | 170 | 127 | 260 | 177 | 350 | 227 | 440 |
| 32 | 90 | 82 | 180 | 132 | 270 | 182 | 360 | 232 | 450 |
| 38 | 100 | 88 | 190 | 138 | 280 | 188 | 370 | 238 | 460 |
| 43 | 110 | 93 | 200 | 143 | 290 | 193 | 380 | 243 | 470 |
| 49 | 120 | 99 | 210 | 149 | 300 | 199 | 390 | 249 | 480 |
| 54 | 130 | 104 | 220 | 154 | 310 | 204 | 400 | 254 | 490 |

##### EMERGENCY PROCEDURES

AIRSPEEDS FOR EMERGENCY OPERATIONS

Engine Failure after Takeoff:

Wing Flaps Up …………………………	80 Kts

Wing Flaps Down ……………………..	70 Kts

Maneuvering Speed (Va) ……………………….	115 Kts

Maximum Glide ………………………………...80 Kts

ELECTRICAL / ALTERNATOR FAILURE

- Avionics –OFF

- Master Switch – OFF, both sides

- Master Switch – ON

IF ALTERNATOR IS STILL OFF-LINE:

- Master Switch – OFF

- All Electrical Switches – OFF

- Alternator Breaker – PULL

- Avionics – OFF

- Avionics – ON as required

- Electrical Equipment – ON, as required

- Flight – TERMINATE as soon as practical, aircraft is on battery reserves only.

IF ALTERNATOR CIRCUIT BREAKER TRIPED:

- Master Switch – OFF

- All Electrical Switches – OFF

- Avionics – OFF

- WAIT 5 minutes for breaker cool down

- Alternator Breaker – RESET ONCE

- Avionics – ON as required

- Electrical Equipment – ON, as required

- Flight – TERMINATE as soon as practical

EMERGENCY PROCEDURES

ENGINE FAILURES

ENGINE FAILURE DURING TAKEOFF RUN

- Throttle –IDLE

- Brakes – APPLY

- Wing Flaps – RETRACT

- Mixture – IDLE CUT-OFF

- Ignition Switch – OFF

- Master Switch – OFF

ENGINE FAILURE IMMEDIATELY AFTER TAKEOFF

- Airspeed – 70 KIAS

- Mixture – IDLE CUT-OFF

- Fuel Selector Valve – OFF

- Ignition Switch – OFF

- Wing Flaps – AS REQUIRED

- Master Switch – OFF

ENGINE FAILURE DURING FLIGHT

- Airspeed –80 Knots

- Boost Pump – ON

- Fuel Selector – SWITCH TANKS

- Mixture – RICH

- Carb heat -- ON

- Ignition Switch – BOTH, LEFT, RIGHT

- Transponder – 7700

- Radio --MAYDAY 121.5

ICING

- Carb heat -- ON

- Cabin heat – MAX

- Heading – MAKE 180 degree turn

- Altitude -- CHANGE

- Throttle – FULL

- Flaps – UP

- Land -- IMMEDIATELY

EMERGENCY PROCEDURES

FIRES

DURING START ON GROUND

- Cranking – CONTINUE, to get a start which would suck the flames and accumulated fuel through the carb and into the engine.

If engine starts:

- Power – 1700 RPM for a few minutes

- Engine – SHUTDOWN and inspect for damage

If engine fails to start:

- Throttle – FULL OPEN

- Mixture – IDLE CUT-OFF

- Cranking – CONTINUE

- Fire Extinguisher – ACTIVATE behind seat

- Engine – SECURE

ENGINE FIRE IN FLIGHT

- Mixture – IDLE CUT-OFF

- Fuel Selector Valve – OFF

- Master Switch – OFF

- Cabin Heat and Air – OFF

- Radio – MAYDAY 121.5

ELECTRICAL FIRE IN FLIGHT

- Master Switch – OFF

- Avionics – OFF

- All Other Switches (except ignition) – OFF

- Vents/ Cabin Air/ Heat – CLOSED

- Fire Extinguisher – ACTIVATE Behind seat

CABIN FIRE

- Master Switch – OFF

- Vents/ Cabin Heat – CLOSED

- Fire Extinguisher – ACTIVATE behind seat

WING FIRE

- Nav & Strobe Lights – OFF

- Landing Light – OFF

91.125 - ATC light signals.

ATC light signals have the meaning shown in the following table:

| Color and type of signal | Meaning with respect to aircraft on the surface | Meaning with respect to aircraft in flight |
| --- | --- | --- |
| Steady green | Cleared for takeoff | Cleared to land |
| Flashing green | Cleared to taxi | Return for landing |
| Steady red | Stop | Give way to other aircraft and continue circling. |
| Flashing red | Taxi clear of runway in use | Airport unsafe—do not use |
| Flashing white | Return to starting point on airport | N/A |
| Alternating red and green | Exercise extreme caution | Exercise extreme caution |

## Compass Headings, VFR under 18,000ft

| Course | Altitude |
| --- | --- |
| 0 – 179 degrees | Odd thousand +500 |
| 180 – 360 degrees | Even thousand + 500 |

Squawk Codes

1200 	Visual Flight Rules (VFR)

7500 	Hijack

7600	Communications failure

7700	Emergency